## FEATURES

1. Low impedance for high frequency.

高频低阻抗。

2. Radio type for switching power supply and anti-solvent.

广泛应用于开关电源等装置中。

SPECIFICATIONS    规格表

| Item 项目 | Performance characteristics 主要特性 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated Working Voltage Range额定工作电压范围 | 6.3 to 100V |  |  |  |  |  | 160 to 450V |  |  |  |  |  |  |  |
| Operation Temperature Range 使用温度范围 | -40 to +85℃ |  |  |  |  |  | -25 to +85℃ |  |  |  |  |  |  |  |
| Nominal Capacitance Range 标称容量 | 0.47 to 15000μF |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacitance Tolerance 电容量允许偏差 | ±20%(120Hz,+20℃) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leakage Current 漏电流 | I≤0.03CV or 4(μA) after 1 minutes whichever is greater measured with rated working voltage applied at +20℃ 20℃.升到额定电压后两分钟读数.取较大者. |  |  |  |  |  | CV≤1000:I=0.1CV +40(μA) max. CV＞1000:I=0.04CV +100(μA) max. after 1 minutes application of rated working voltage at +20℃ 20℃.升到额定电压后两分钟读数. |  |  |  |  |  |  |  |
| Dissipation Factor tanδ (120Hz,+20℃) 损耗角正切值tanδ | Working voltage(V) | 6.3 | 10 |  | 16 |  |  | 25 | 35 | 50 |  | 63 |  | 100 |
|  | tanδ(max.) | 0.22 | 0.19 |  | 0.16 |  |  | 0.14 | 0.12 | 0.10 |  | 0.08 |  | 0.08 |
|  | Working voltage(V) | 160 |  | 200 |  | 250 |  |  | 350 |  | 400 |  | 450 |  |
|  | tanδ(max.) | 0.20 |  | 0.20 |  | 0.20 |  |  | 0.20 |  | 0.25 |  | 0.25 |  |
| Low Temperature Characteristic 低温特性 | Impedance ratio max. at 120Hz 120Hz时的最大阻抗比率. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Working voltage(V) | 6.3 | 10 |  | 16 |  |  | 25 | 35 | 50 |  | 63 |  | 100 |
|  | Z-55℃/ Z+20℃ | 5 | 5 |  | 5 |  |  | 4 | 4 | 4 |  | 4 |  | 4 |
|  | Working voltage(V) | 160 |  | 200 |  | 250 |  |  | 350 |  | 400 |  | 450 |  |
|  | Z-55℃/ Z+20℃ | 4 |  | 4 |  | 7 |  |  | 7 |  | 7 |  | 15 |  |
| High Temperature Loading 高温负荷 | Test conditions 试验条件 Duration : 2000 hours Ambient temperature : +85℃ Applied voltage : Rated DC working voltage. Post test requirements at +20℃ 试验完毕20℃测试应满足 Leakage current : ≤Initial specified value Capacitance change : ≤±20% of initial measured value tan δ : ≤150% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shelf Life 高温贮存 Shelf Life 高温贮存 | Test conditions 试验条件 Duration : 1000 hours Ambient temperature : +85℃ Applied voltage : (None) 试验完毕20℃测试应满足 Leakage current : ≤Initial specified value Capacitance change : ≤±20% of initial measured value tan δ : ≤150% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Others 其它 | JIS C-5141 JIS C-5102 |  |  |  |  |  |  |  |  |  |  |  |  |  |

## CASE SIZE TABLE

# DIMENSIONS

| Volage Cap.( μF) | 6.3V | 10V | 16V | 25V | 35V |
| --- | --- | --- | --- | --- | --- |
| 22 |  |  |  |  | 5×11 |
| 27 |  |  |  |  | 5×11 |
| 33 |  |  |  | 5×11 | 6.3×11 |
| 39 |  |  |  | 5×11 | 6.3×11 |
| 47 |  |  | 5×11 | 6.3×11 | 6.3×11 |
| 56 |  |  | 5×11 | 6.3×11 | 6.3×11 |
| 68 |  | 5×11 | 6.3×11 | 6.3×11 | 8×12 |
| 82 |  | 5×11 | 6.3×11 | 6.3×11 | 8×12 |
| 100 | 5×11 | 6.3×11 | 6.3×11 | 8×12 | 8×12 |
| 120 | 5×11 | 6.3×11 | 6.3×11 | 8×12 | 8×15 |
| 150 | 6.3×11 | 6.3×11 | 8×12 | 8×12 | 8×15 |
| 180 | 6.3×11 | 6.3×11 | 8×12 | 8×15 | 8×20 |
| 220 | 6.3×11 | 8×12 | 8×12 | 8×15 | 8×20 |
| 270 | 8×12 | 8×12 | 8×15 | 8×20 | 10×20 |
| 330 | 8×12 | 8×12 | 8×15 | 8×20 | 10×20 |
| 390 | 8×12 | 8×15 | 8×20 | 10×20 | 10×25 |
| 470 | 8×15 | 8×15 | 8×20 | 10×20 | 10×30 |
| 560 | 8×15 | 8×20 | 10×20 | 10×25 | 13×20 |
| 680 | 8×20 | 8×20 | 10×20 | 10×30 | 13×25 |
| 820 | 8×20 | 10×20 | 10×25 | 13×20 | 13×25 |
| 1000 | 10×20 | 10×20 | 10×30 | 13×25 | 13×30 |
| 1200 | 10×20 | 10×25 | 13×20 | 13×25 | 13×35 |
| 1500 | 10×25 | 10×30 | 13×25 | 13×30 | 13×40 |
| 1800 | 10×30 | 10×30 | 13×30 | 13×35 | 16×30 |
| 2200 | 10×30 | 13×20 | 13×30 | 13×40 | 16×35 |
| 2700 | 13×25 | 13×25 | 13×35 | 16×30 | 16×40 |
| 3300 | 13×25 | 13×30 | 13×40 | 16×35 | 18×40 |
| 3900 | 13×30 | 13×35 | 16×30 | 16×40 |  |
| 4700 | 13×35 | 13×40 | 16×35 | 18×40 |  |
| 5600 | 13×40 | 16×30 | 16×40 |  |  |
| 6800 | 16×30 | 16×35 | 18×35 |  |  |
| 8200 | 16×35 | 16×40 | 18×40 |  |  |
| 10000 | 16×40 | 18×40 |  |  |  |
| 12000 | 18×35 |  |  |  |  |
| 15000 | 18×40 |  |  |  |  |

# DIMESIONS

| Volage Cap.( μF) | 50V | 63V | 80V | 100V |
| --- | --- | --- | --- | --- |
| 0.47 | 5×11 |  |  | 5×11 |
| 0.68 | 5×11 |  |  | 5×11 |
| 1 | 5×11 |  |  | 5×11 |
| 1.5 | 5×11 |  |  | 5×11 |
| 2.2 | 5×11 |  |  | 5×11 |
| 3.3 | 5×11 |  |  | 5×11 |
| 4.7 | 5×11 |  | 5×11 | 6.3×11 |
| 6.8 | 5×11 |  | 5×11 | 6.3×11 |
| 10 | 5×11 | 5×11 | 6.3×11 | 6.3×11 |
| 12 | 5×11 | 5×11 | 6.3×11 | 6.3×11 |
| 15 | 5×11 | 6.3×11 | 6.3×11 | 8×12 |
| 18 | 5×11 | 6.3×11 | 6.3×11 | 8×12 |
| 22 | 6.3×11 | 6.3×11 | 8×12 | 8×12 |
| 27 | 6.3×11 | 6.3×11 | 8×12 | 8×15 |
| 33 | 6.3×11 | 8×12 | 8×12 | 8×15 |
| 39 | 6.3×11 | 8×12 | 8×15 | 8×20 |
| 47 | 8×12 | 8×12 | 8×15 | 10×20 |
| 56 | 8×12 | 8×15 | 8×20 | 10×20 |
| 68 | 8×12 | 8×15 | 10×20 | 10×25 |
| 82 | 8×15 | 8×20 | 10×20 | 10×30 |
| 100 | 8×20 | 10×20 | 10×25 | 10×30 |
| 120 | 8×20 | 10×20 | 10×30 | 13×25 |
| 150 | 10×20 | 10×25 | 10×30 | 13×25 |
| 180 | 10×20 | 10×30 | 13×25 | 13×30 |
| 220 | 10×25 | 13×20 | 13×30 | 13×35 |
| 270 | 10×30 | 13×25 | 13×30 | 13×40 |
| 330 | 10×30 | 13×25 | 13×35 | 16×30 |
| 390 | 13×25 | 13×30 | 13×40 | 16×35 |
| 470 | 13×25 | 13×35 | 16×30 | 16×40 |
| 560 | 13×30 | 13×40 | 16×35 | 18×35 |
| 680 | 13×35 | 16×30 | 16×40 | 18×40 |
| 820 | 13×40 | 16×35 | 18×35 |  |
| 1000 | 16×30 | 16×40 | 18×40 |  |
| 1200 | 16×35 | 18×35 |  |  |
| 1500 | 16×40 | 18×40 |  |  |
| 1800 | 18×35 |  |  |  |
| 2200 | 18×40 |  |  |  |

| Volage Cap.( μF) | 160V |  | 200V |  | 250V |  | 315V |  | 350V |  | 400V |  | 450V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8×12 | 27 | 8×12 | 27 | 8×12 | 27 | 8×12 | 27 | 10×12 | 20 | 10×12 | 24 | 10×16 | 24 |
| 2.2 | 8×12 | 42 | 8×12 | 42 | 10×12 | 45 | 10×12 | 45 | 10×16 | 48 | 10×16 | 39 | 10×20 | 39 |
| 3.3 | 10×12 | 70 | 10×12 | 70 | 10×16 | 73 | 10×16 | 73 | 10×20 | 75 | 10×20 | 66 | 13×20 | 67 |
| 4.7 | 10×12 | 80 | 10×16 | 80 | 10×16 | 84 | 10×20 | 91 | 13×20 | 91 | 13×20 | 77 | 13×25 | 77 |
| 10 | 10×16 | 126 | 10×20 | 126 | 13×20 | 137 | 13×20 | 137 | 13×25 | 140 | 13×25 | 119 | 16×25 | 126 |
| 22 | 13×20 | 196 | 13×25 | 190 | 16×25 | 210 | 16×25 | 210 | 16×25 | 210 | 16×30 | 182 | 16×35 | 189 |
| 33 | 13×25 | 245 | 16×25 | 245 | 16×25 | 252 | 16×30 | 259 | 16×35 | 266 | 18×35 | 238 | 18×40 | 238 |
| 47 | 16×25 | 308 | 16×25 | 308 | 16×30 | 315 | 18×35 | 329 | 18×40 | 336 |  |  |  |  |
| 100 | 16×35 | 462 | 18×40 | 462 | 18×40 | 483 |  |  |  |  |  |  |  |  |

## STANDARD RATINGS

| Voltage(Code) Cap.( μF) | 6.3V |  |  |  | 10V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case size φD×L(mm | mpedance(ΩMAX.) | Allowable rippler (mA rms) |  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  |
|  |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |
| 68 |  |  |  |  | 5×11 | 1.30 | 217 | 136 |
| 82 |  |  |  |  | 5×11 | 1.10 | 245 | 154 |
| 100 | 5×11 | 1.40 | 210 | 139 | 6.3×11 | 0.84 | 294 | 189 |
| 120 | 5×11 | 1.10 | 245 | 161 | 6.3×11 | 0.72 | 329 | 224 |
| 150 | 6.3×11 | 0.78 | 315 | 217 | 6.3×11 | 0.55 | 372 | 259 |
| 180 | 6.3×11 | 0.60 | 350 | 245 | 6.3×11 | 0.46 | 406 | 287 |
| 220 | 6.3×11 | 0.48 | 399 | 287 | 8×12 | 0.38 | 528 | 378 |
| 270 | 8×12 | 0.39 | 518 | 385 | 8×12 | 0.31 | 567 | 420 |
| 330 | 8×12 | 0.32 | 567 | 434 | 8×12 | 0.26 | 644 | 490 |
| 390 | 8×12 | 0.27 | 623 | 483 | 8×15 | 0.22 | 770 | 602 |
| 470 | 8×15 | 0.22 | 770 | 609 | 8×15 | 0.19 | 833 | 665 |
| 560 | 8×15 | 0.19 | 833 | 672 | 8×20 | 0.16 | 1022 | 826 |
| 680 | 8×20 | 0.16 | 1023 | 847 | 8×20 | 0.13 | 1113 | 924 |
| 820 | 8×20 | 0.13 | 1113 | 938 | 10×20 | 0.11 | 1379 | 1169 |
| 1000 | 10×20 | 0.12 | 1330 | 1148 | 10×20 | 0.096 | 1484 | 1281 |
| 1200 | 10×20 | 0.10 | 1428 | 1253 | 10×25 | 0.078 | 1792 | 1568 |
| 1500 | 10×25 | 0.084 | 1708 | 1526 | 10×30 | 0.072 | 2016 | 1806 |
| 1800 | 10×30 | 0.078 | 1918 | 1722 | 13×20 | 0.057 | 2058 | 1848 |
| 2200 | 13×25 | 0.066 | 2058 | 1848 | 13×25 | 0.045 | 2394 | 2142 |
| 2700 | 13×25 | 0.051 | 2226 | 2002 | 13×30 | 0.036 | 2716 | 2436 |
| 3300 | 13×30 | 0.045 | 2394 | 2142 | 13×35 | 0.032 | 3052 | 2744 |
| 4700 | 13×35 | 0.034 | 2674 | 2646 | 16×30 | 0.028 | 3388 | 3038 |
| 5600 | 13×40 | 0.031 | 2940 | 2850 | 16×35 | 0.026 | 3654 | 3276 |
| 6800 | 16×30 | 0.029 | 3178 | 2982 | 16×35 | 0.024 | 3752 | 3374 |
| 8200 | 16×35 | 0.027 | 3318 | 3206 | 16×40 | 0.023 | 3948 | 3542 |
| 10000 | 16×40 | 0.025 | 3570 | 3158 | 18×40 | 0.021 | 4256 | 3824 |
| 12000 | 18×35 | 0.023 | 3948 | 3542 |  |  |  |  |
| 15000 | 18×40 | 0.022 | 4144 | 3724 |  |  |  |  |

## STANDARD RATINGS

| Voltage(Code) Item Cap.(uf) Code | 16V |  |  |  | 25V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  |
|  |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |
| 33 |  |  |  |  | 5×11 | 1.30 | 217 | 123 |
| 39 |  |  |  |  | 5×11 | 1.10 | 245 | 140 |
| 47 | 5×11 | 1.30 | 217 | 127 | 6.3×11 | 0.84 | 294 | 175 |
| 56 | 5×11 | 1.10 | 245 | 147 | 6.3×11 | 0.72 | 329 | 196 |
| 68 | 6.3×11 | 0.78 | 308 | 189 | 6.3×11 | 0.57 | 364 | 224 |
| 82 | 6.3×11 | 0.66 | 336 | 217 | 6.3×11 | 0.47 | 399 | 252 |
| 100 | 6.3×11 | 0.55 | 371 | 245 | 8×12 | 0.39 | 578 | 343 |
| 120 | 6.3×11 | 0.45 | 406 | 275 | 8×12 | 0.32 | 587 | 385 |
| 150 | 8×12 | 0.37 | 525 | 364 | 8×12 | 0.26 | 644 | 448 |
| 180 | 8×12 | 0.31 | 567 | 399 | 8×15 | 0.22 | 770 | 546 |
| 220 | 8×12 | 0.26 | 604 | 469 | 8×15 | 0.18 | 875 | 637 |
| 270 | 8×15 | 0.22 | 770 | 574 | 8×20 | 0.15 | 1050 | 784 |
| 330 | 8×15 | 0.18 | 883 | 637 | 8×20 | 0.13 | 1113 | 854 |
| 390 | 8×20 | 0.16 | 1022 | 798 | 10×20 | 0.11 | 1379 | 1078 |
| 470 | 8×20 | 0.14 | 1078 | 861 | 10×20 | 0.10 | 1428 | 1134 |
| 560 | 10×20 | 0.12 | 1330 | 1078 | 10×25 | 0.084 | 1708 | 1386 |
| 680 | 10×20 | 0.10 | 1428 | 1183 | 10×30 | 0.072 | 1588 | 1652 |
| 820 | 10×25 | 0.084 | 1708 | 1442 | 13×20 | 0.059 | 2002 | 1694 |
| 1000 | 10×30 | 0.072 | 1974 | 1694 | 13×25 | 0.048 | 2324 | 2002 |
| 1200 | 13×20 | 0.060 | 2002 | 1750 | 13×25 | 0.043 | 2404 | 2170 |
| 1500 | 13×25 | 0.048 | 2324 | 2086 | 13×30 | 0.035 | 2772 | 2492 |
| 1800 | 13×30 | 0.039 | 2632 | 2366 | 13×35 | 0.032 | 3052 | 2744 |
| 2200 | 13×30 | 0.034 | 2814 | 2520 | 13×40 | 0.029 | 3304 | 2968 |
| 2700 | 13×35 | 0.031 | 3108 | 2796 | 16×30 | 0.027 | 3458 | 3108 |
| 3300 | 13×40 | 0.028 | 3374 | 3034 | 16×35 | 0.025 | 3752 | 3374 |
| 3900 | 16×30 | 0.027 | 3458 | 3108 | 16×40 | 0.023 | 3848 | 3542 |
| 4700 | 16×35 | 0.025 | 3752 | 3374 | 18×40 | 0.022 | 4104 | 3724 |
| 5600 | 16×40 | 0.024 | 3948 | 3542 |  |  |  |  |
| 6800 | 18×35 | 0.022 | 4060 | 3654 |  |  |  |  |
| 8200 | 18×40 | 0.021 | 4256 | 3822 |  |  |  |  |

# STANDARD RATINGS

| Voltage(Code) Item Cap.(uf) Code | 35V |  |  |  | 63V (1J) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  |
|  |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |
| 10 |  |  |  |  | 5×11 | 1.60 | 189 | 94 |
| 12 |  |  |  |  | 5×11 | 1.40 | 203 | 101 |
| 15 |  |  |  |  | 6.3×11 | 1.10 | 259 | 129 |
| 18 |  |  |  |  | 6.3×11 | 0.95 | 273 | 140 |
| 22 | 5×11 | 1.30 | 224 | 119 | 6.3×11 | 0.78 | 301 | 154 |
| 27 | 5×11 | 1.00 | 252 | 189 | 6.3×11 | 0.64 | 336 | 182 |
| 33 | 6.3×11 | 0.78 | 315 | 195 | 8×12 | 0.52 | 427 | 238 |
| 39 | 6.3×11 | 0.66 | 343 | 196 | 8×12 | 0.45 | 462 | 266 |
| 47 | 6.3×11 | 0.54 | 378 | 234 | 8×12 | 0.37 | 511 | 301 |
| 56 | 6.3×11 | 0.45 | 413 | 252 | 8×15 | 0.31 | 630 | 385 |
| 68 | 8×12 | 0.37 | 518 | 322 | 8×15 | 0.26 | 700 | 441 |
| 82 | 8×12 | 0.31 | 581 | 371 | 8×20 | 0.22 | 840 | 539 |
| 100 | 8×12 | 0.26 | 644 | 427 | 10×20 | 0.18 | 1050 | 693 |
| 120 | 8×15 | 0.22 | 770 | 508 | 10×20 | 0.15 | 1148 | 777 |
| 150 | 8×15 | 0.18 | 833 | 581 | 10×25 | 0.13 | 1330 | 931 |
| 180 | 8×20 | 0.16 | 1022 | 728 | 10×30 | 0.11 | 1554 | 1106 |
| 220 | 8×20 | 0.13 | 1113 | 812 | 13×20 | 0.094 | 1516 | 1169 |
| 270 | 10×20 | 0.11 | 1329 | 1089 | 13×25 | 0.081 | 1816 | 1409 |
| 330 | 10×20 | 0.096 | 1484 | 1134 | 13×25 | 0.072 | 1988 | 1526 |
| 390 | 10×25 | 0.084 | 1708 | 1337 | 13×30 | 0.059 | 2268 | 1764 |
| 470 | 10×30 | 0.072 | 1988 | 1582 | 13×35 | 0.052 | 2492 | 1988 |
| 560 | 13×20 | 0.059 | 2002 | 1624 | 13×40 | 0.047 | 2730 | 2122 |
| 680 | 13×25 | 0.048 | 2324 | 1918 | 16×30 | 0.043 | 2870 | 2380 |
| 820 | 13×25 | 0.042 | 2464 | 2080 | 16×35 | 0.040 | 3108 | 2632 |
| 1000 | 13×30 | 0.035 | 2772 | 2394 | 16×40 | 0.037 | 3318 | 2870 |
| 1200 | 13×35 | 0.031 | 3052 | 2688 | 18×40 | 0.034 | 3514 | 3094 |
| 1500 | 13×40 | 0.029 | 3304 | 2968 |  |  |  |  |
| 1800 | 16×30 | 0.027 | 3458 | 3108 |  |  |  |  |
| 2200 | 16×35 | 0.024 | 3752 | 3374 |  |  |  |  |
| 2700 | 16×40 | 0.022 | 4060 | 3654 |  |  |  |  |
| 300 | 18×40 | 0.021 | 4256 | 3822 |  |  |  |  |

# STANDARD RATINGS

| Voltage(Code) Item Cap.(uf) Code | 50V |  |  |  | 100V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  | Case size φD×L(mm) | Impedance(ΩMAX.) | Allowable rippler (mA rms) |  |
|  |  | 20℃/100KHZ | 85℃/10KHZ~ 200KHZ | 85℃/120HZ |  | 20℃/100KHZ | 8℃/10KHZ~ 200KHZ | 85℃/120HZ |
| 0.47 | 5×11 | 31.0 | 31 | 15 | 5×11 | 43.0 | 24 | 10 |
| 0.68 | 5×11 | 21.0 | 39 | 20 | 5×11 | 23.0 | 32 | 14 |
| 1 | 5×11 | 14.0 | 50 | 25 | 5×11 | 17.0 | 38 | 16 |
| 1.5 | 5×11 | 9.80 | 63 | 31 | 5×11 | 10.0 | 49 | 22 |
| 2.2 | 5×11 | 6.50 | 76 | 38 | 5×11 | 6.60 | 60 | 28 |
| 3.3 | 5×11 | 4.60 | 92 | 46 | 5×11 | 4.10 | 76 | 35 |
| 4.7 | 5×11 | 3.10 | 113 | 56 | 6.3×11 | 2.80 | 95 | 44 |
| 6.8 | 5×11 | 2.50 | 127 | 63 | 6.3×11 | 1.90 | 116 | 52 |
| 10 | 5×11 | 2.00 | 161 | 80 | 6.3×11 | 1.20 | 146 | 66 |
| 12 | 5×11 | 1.70 | 175 | 87 | 6.3×11 | 1.00 | 160 | 72 |
| 15 | 5×11 | 1.30 | 203 | 101 | 8×12 | 0.81 | 183 | 82 |
| 18 | 5×11 | 1.10 | 217 | 111 | 8×12 | 0.67 | 217 | 92 |
| 22 | 6.3×11 | 0.91 | 273 | 140 | 8×12 | 0.55 | 322 | 154 |
| 27 | 6.3×11 | 0.74 | 301 | 161 | 8×15 | 0.47 | 377 | 184 |
| 33 | 6.3×11 | 0.60 | 336 | 188 | 8×15 | 0.38 | 419 | 214 |
| 39 | 6.3×11 | 0.50 | 364 | 240 | 8×20 | 0.33 | 493 | 255 |
| 47 | 8×12 | 0.42 | 463 | 273 | 10×20 | 0.28 | 592 | 320 |
| 56 | 8×12 | 0.35 | 504 | 308 | 10×20 | 0.24 | 638 | 396 |
| 68 | 8×12 | 0.28 | 574 | 357 | 10×25 | 0.21 | 736 | 411 |
| 82 | 8×15 | 0.22 | 708 | 448 | 10×30 | 0.18 | 848 | 490 |
| 100 | 8×20 | 0.18 | 868 | 574 | 10×30 | 0.15 | 928 | 553 |
| 120 | 8×20 | 0.16 | 938 | 637 | 13×25 | 0.13 | 1084 | 657 |
| 150 | 10×20 | 0.13 | 1188 | 798 | 13×25 | 0.11 | 1117 | 704 |
| 180 | 10×20 | 0.11 | 1246 | 859 | 13×30 | 0.098 | 1266 | 817 |
| 220 | 10×25 | 0.098 | 1456 | 1068 | 13×35 | 0.087 | 1400 | 922 |
| 270 | 10×30 | 0.085 | 1680 | 1260 | 13×40 | 0.072 | 1582 | 1070 |
| 330 | 10×30 | 0.072 | 1820 | 1393 | 16×30 | 0.062 | 2128 | 1462 |
| 390 | 13×25 | 0.053 | 2016 | 1568 | 16×35 | 0.053 | 2422 | 1688 |
| 470 | 13×25 | 0.048 | 2188 | 1666 | 16×40 | 0.047 | 2688 | 1892 |
| 560 | 13×30 | 0.040 | 2353 | 1904 | 18×35 | 0.41 | 2898 | 2117 |
| 680 | 13×35 | 0.036 | 2590 | 2143 | 18×40 | 0.036 | 3220 | 2407 |
| 820 | 13×40 | 0.033 | 2814 | 2380 |  |  |  |  |
| 1000 | 16×30 | 0.030 | 2968 | 2562 |  |  |  |  |
| 1200 | 16×35 | 0.028 | 3764 | 2786 |  |  |  |  |
| 1500 | 16×40 | 0.026 | 3374 | 3038 |  |  |  |  |
| 1800 | 18×35 | 0.025 | 3444 | 3054 |  |  |  |  |
| 2200 | 18×40 | 0.024 | 3584 | 3220 |  |  |  |  |