# FEATURES

- Superminiture, 5mmL, high temperature, suit for use in electronic circuit of high density assembly of videocorder, remote controller, VCD etc.

超小体积、5mm高、高温，广泛应用于VCD、遥控器等高密度组装的电子线路中。

SPECIFICATIONS    规格表

| Item 项目 | Performance characteristics 主要特性 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated Working Voltage Range 额定工作电压范围 | 4to 50V |  |  |  |  |  |  |  |
| Operation Temperature Range 使用温度范围 | -40 to +105℃ |  |  |  |  |  |  |  |
| Nominal Capacitance Range 标称容量 | 0.1 to 100μF |  |  |  |  |  |  |  |
| Capacitance Tolerance 电容量允许偏差 | ±20%(120Hz,+20℃) |  |  |  |  |  |  |  |
| Leakage Current 漏电流 | 1≤0.01CV or 3(μA) which is greater measured after 2 minutes application of rated working voltage at +20℃ 20℃升到额定电压后两分钟读数，取较大者. |  |  |  |  |  |  |  |
| Dissipation Factor tanδ (120Hz,+20℃) 损耗角正切值tanδ | Working voltage(V) | 4 | 6.3 | 10 | 16 | 25 | 35 | 50 |
|  | tanδ(max.) | 0.37 | 0.28 | 0.24 | 0.20 | 0.16 | 0.13 | 0.12 |
| Low Temperature Characteristics 低温特性 | Impedance ratio max. at 120Hz 120Hz时的最大阻抗比率. |  |  |  |  |  |  |  |
|  | Working voltage(V) | 4 | 6.3 | 10 | 16 | 25 | 35 | 50 |
|  | Z-25℃/ Z+20℃ | 6 | 3 | 3 | 2 | 2 | 2 | 2 |
|  | Z-40℃/ Z+20℃ | 12 | 8 | 5 | 4 | 3 | 3 | 3 |
| High Temperature loading 高温负荷 | Test conditions 测试条件 Duration : 1000 hours Ambient temperature : +105℃ Applied voltage : Rated DC working voltage Post test requirements at +20℃ 试验完毕在20℃测试应满足 Leakage current : ≤Initial specified value Capacitance change : ≤±20% of initial measured value (4V: ≤±30%) tanδ : ≤200% of initial specified value |  |  |  |  |  |  |  |
| Shelf Life 高温贮存 | Test conditions Post test requirements at +20℃ 测试条件 Duration : 1000 hours Ambient temperature : +105℃ Applied voltage : (None) Post test requirements at +20℃ 试验完毕在20℃测试应满足. Leakage current : ≤200%Initial specified value Capacitance change : ≤±20% of initial measured value (4V: ≤±30%) tanδ : ≤200% of initial specified value |  |  |  |  |  |  |  |
| Others 其它 | JIS C – 5141 JIS C - 5102 |  |  |  |  |  |  |  |

CASE SIZE TABLE    外形尺寸图

DIMESIONS                                                               φD×L(mm)

| Voltage Cap.(μF) | 4V |  | 6.3V |  | 10V |  | 16V |  | 25V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 |  |  |  |  |  |  |  |  |  |  |
| 0.22 |  |  |  |  |  |  |  |  |  |  |
| 0.33 |  |  |  |  |  |  |  |  |  |  |
| 0.47 |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2.2 |  |  |  |  |  |  |  |  |  |  |
| 3.3 |  |  |  |  |  |  |  |  |  |  |
| 4.7 |  |  |  |  |  |  |  |  | 4×5 | 13 |
| 10 |  |  |  |  |  |  | 4×5 | 18 | 5×5 | 23 |
| 22 | 4×5 | 22 | 4×5 | 22 | 5×5 | 27 | 5×5 | 30 | 6.3×5 | 38 |
| 33 | 5×5 | 30 | 5×5 | 30 | 5×5 | 35 | 6.3×5 | 40 | 6.3×5 | 48 |
| 47 | 5×5 | 36 | 5×5 | 36 | 6.3×5 | 46 | 6.3×5 | 50 |  |  |
| 100 | 6.3×5 | 60 | 6.3×5 | 60 |  |  |  |  |  |  |

Maximum Allowable Ripple Current (mA rms)at 85℃,120Hz

| Voltage Cap.(μF) | 35V |  | 50V |  |
| --- | --- | --- | --- | --- |
| 0.1 |  |  | 4×5 | 1.0 |
| 0.22 |  |  | 4×5 | 2.6 |
| 0.33 |  |  | 4×5 | 3.2 |
| 0.47 |  |  | 4×5 | 3.8 |
| 1 |  |  | 4×5 | 6.2 |
| 2.2 | 4×5 | 7 | 4×5 | 11 |
| 3.3 | 4×5 | 11 | 4×5 | 14 |
| 4.7 | 4×5 | 15 | 5×5 | 19 |
| 10 | 5×5 | 25 | 6.3×5 | 30 |
| 22 | 6.3×5 | 48 |  |  |
| 33 |  |  |  |  |
| 47 |  |  |  |  |
| 100 |  |  |  |  |

Maximum Allowable Ripple Current (mA rms)at 105℃,120Hz