FEATURES  特点

- Rated Working Voltage Range 6.3 to 100VCD/160 to 450VCD at Operation Temperature Range –40 to +105℃/-25

to +105℃.

额定工作电压及工作环境温度范围分别为6.3~100VDC/–40+105℃，160~450VDC/-25+105℃.

- This series is for communication equipments, switching power supply, industrial measuring instruments, automotive electric products, etc.

适应于通讯设备、开关电源、工业测试机及各种自动化电子产品中。

SPECIFICATIONS   规格表

| Item 项目 | Performance characteristics 主要特性 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated Working Voltage Range 额定工作电压范围 | 6.3 to 100V DC |  |  |  |  |  |  |  |  |  |  |  |  | 160 to 450V DC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation Temperature Range 使用温度范围 | -40 to +105℃ |  |  |  |  |  |  |  |  |  |  |  |  | -25 to +105℃ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal Capacitance Range 标称容量 | 0.1 to 33000μF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacitance Tolerance 电容量允许偏差 | ±20%(120Hz,+20℃) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leakage Current 低温特性 | 1≤0.01CV or 3(μA) after 2 minutes whichever is greater measured with rated working voltage applied at +20℃ 20℃.升到额定电压后两分钟时读数.取较大者. |  |  |  |  |  |  |  |  |  |  |  |  | 1≤0.03CV +40(μA) after 2 minutes application of rated working voltage at +20℃ 20℃.升到额定电压后两分钟时读数. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dissipation Factor tanδ (120Hz,+20℃) 损耗角正切值tanδ | Working voltage(V) 工作电压 |  | 6.3 |  | 10 |  |  |  |  | 16 |  |  |  |  |  | 25 |  | 35 |  |  | 50 |  |  |  |  | 63 |  |  |  |  | 100 |  |
|  | tanδ(max.) |  | 0.28 |  | 0.24 |  |  |  |  | 0.20 |  |  |  |  |  | 0.16 |  | 0.14 |  |  | 0.12 |  |  |  |  | 0.10 |  |  |  |  | 0.08 |  |
|  | Working voltage(V) 工作电压 |  | 160 |  |  |  | 200 |  |  |  |  | 250 |  |  |  |  |  | 350 |  |  |  |  | 400 |  |  |  |  | 450 |  |  |  |  |
|  | tanδ(max.) |  | 0.20 |  |  |  | 0.20 |  |  |  |  | 0.20 |  |  |  |  |  | 0.25 |  |  |  |  | 0.25 |  |  |  |  | 0.25 |  |  |  |  |
|  | For capacitance value >1000μF, add 0.02 per another 1000μF 对容量大于1000μF者，每增加1000μF，DF增加0.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ripple Current 纹波电流 | Refer to standard products table (120Hz,85℃) 专指标准产品(120Hz,85℃). Correction factor for frequency 标准产品频率调整系数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Voltage(V) | Capacitance Range(μF) |  |  |  |  |  |  | 50Hz |  |  |  |  |  | 120Hz |  |  |  |  | 300Hz |  |  |  |  | 1kHz |  |  |  |  | 10kHz～ |  |  |
|  | 6.3~100 | ~47 |  |  |  |  |  |  | 0.75 |  |  |  |  |  | 1.00 |  |  |  |  | 1.35 |  |  |  |  | 1.57 |  |  |  |  | 2.00 |  |  |
|  |  | 100~470 |  |  |  |  |  |  | 0.80 |  |  |  |  |  | 1.00 |  |  |  |  | 1.23 |  |  |  |  | 1.34 |  |  |  |  | 1.50 |  |  |
|  |  | 1000~33000 |  |  |  |  |  |  | 0.85 |  |  |  |  |  | 1.00 |  |  |  |  | 0.10 |  |  |  |  | 0.13 |  |  |  |  | 1.15 |  |  |
|  | 160~450 | 0.47~220 |  |  |  |  |  |  | 0.80 |  |  |  |  |  | 1.00 |  |  |  |  | 0.25 |  |  |  |  | 1.40 |  |  |  |  | 1.60 |  |  |
|  |  | 330~1000 |  |  |  |  |  |  | 0.90 |  |  |  |  |  | 1.00 |  |  |  |  | 0.10 |  |  |  |  | 0.13 |  |  |  |  | 1.15 |  |  |
| Low Temperature Characteristics 低温特性 Low Temperature Characteristics 低温特性 | Impedance ratio max. at 120Hz 120Hz时的阻抗比率. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Working voltage(V) |  |  | 6.3 |  | 10 |  |  |  |  | 16 |  |  |  |  |  | 25 |  | 35 |  |  | 50 |  |  |  |  | 63 |  |  |  |  | 100 |
|  | Z-25℃/ Z+20℃ |  |  | 5 |  | 4 |  |  |  |  | 3 |  |  |  |  |  | 2 |  | 2 |  |  | 2 |  |  |  |  | 2 |  |  |  |  | 2 |
|  | Z-40℃/ Z+20℃ |  |  | 10 |  | 8 |  |  |  |  | 6 |  |  |  |  |  | 4 |  | 3 |  |  | 3 |  |  |  |  | 3 |  |  |  |  | 3 |
|  | Working voltage(V) |  |  | 160 |  |  |  | 200 |  |  |  |  | 250 |  |  |  |  |  | 350 |  |  |  |  | 400 |  |  |  |  | 450 |  |  |  |
|  | Z-25℃/ Z+20℃ |  |  | 3 |  |  |  | 3 |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  | 6 |  |  |  |  | 15 |  |  |  |
|  | For capacitance value>1000μF, Add 0.5 per another 1000μF for Z-25℃/ Z+20℃ Add 1.0 per another 1000μF for Z-40℃/ Z+20 对容量大于1000μF者，每增加1000μF. Z-25℃/ Z+20℃增加0.5 对容量大于1000μF者，每增加1000μF. Z-40℃/ Z+20增加1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High Temperature Loading 高温负荷 | Test conditions 试验条件 Duration : 1000 hours Ambient temperature : +105℃ Applied voltage : DC voltage with maximum permissible ripple current specified at +105℃(Sum of the DC voltage and super-imposed peak AC voltage for maximum permissible ripple current should be equal to rated DC working voltage). 105℃时施加最大允许纹波电流的直流电压 (施加的直流电压与最大纹波电流下对应的交流峰值电压之和不得超过电容器的额定工作电压)。 Post test requirements at +20℃ 试验完毕在20℃测试应满足 Leakage current : ≤Initial specified value Capacitance change : ≤±20% of initial measured value tan δ : ≤200% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shelf Life 贮存寿命 | Test conditions 试验条件 Duration : 1000 hours Ambient temperature : +105℃ Applied voltage : (None) Post test requirements at +20℃ 试验完毕在20℃测试应满足 Leakage current : ≤200%Initial specified value Capacitance change : ≤±20% of initial measured value tan δ : ≤200% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Others 其它 | JIS C-5141 JIS C-5102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# CASE SIZE TABLE

DIMESIONS                                                                φD×L(mm)

| Voltage Cap.(μF) | 6.3V |  | 10V |  | 16V |  | 25V |  | 35V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 |  |  |  |  |  |  |  |  |  |  |
| 0.22 |  |  |  |  |  |  |  |  |  |  |
| 0.33 |  |  |  |  |  |  |  |  |  |  |
| 0.47 |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2.2 |  |  |  |  |  |  |  |  |  |  |
| 3.3 |  |  |  |  |  |  |  |  |  |  |
| 4.7 |  |  |  |  |  |  | 5×11 | 22 | 5×11 | 25 |
| 10 |  |  |  |  | 5×11 | 30 | 5×11 | 36 | 5×11 | 39 |
| 22 | 5×11 | 28 | 5×11 | 41 | 5×11 | 54 | 5×11 | 58 | 5×11 | 61 |
| 33 | 5×11 | 43 | 5×11 | 58 | 5×11 | 65 | 5×11 | 68 | 5×11 | 75 |
| 47 | 5×11 | 56 | 5×11 | 68 | 5×11 | 79 | 5×11 | 83 | 6.3×11 | 93 |
| 100 | 5×11 | 93 | 5×11 | 105 | 5×11 | 115 | 6.3×12 | 140 | 8×12 | 150 |
| 220 | 6.3×11 | 145 | 5×12 | 175 | 6.3×12 | 190 | 8×12 | 240 | 10×12.5 | 275 |
| 330 | 6.3×11 | 195 | 6.3×11 | 210 | 8×12 | 265 | 10×13 | 315 | 10×16 | 340 |
| 470 | 6.3×11 | 230 | 6.3×12 | 250 | 8×12 | 315 | 10×13 | 375 | 10×20 | 415 |
| 1000 | 8×12 | 390 | 8×12.5 | 445 | 10×15 | 510 | 10×20 | 595 | 13×20 | 715 |
| 2200 | 10×20 | 645 | 10×20 | 695 | 12×21 | 825 | 13×21 | 930 | 16×25 | 1110 |
| 3300 | 13×20 | 750 | 13×20 | 895 | 13×25 | 1000 | 16×25 | 1180 | 16×35 | 1400 |
| 4700 | 13×20 | 965 | 13×25 | 1080 | 16×25 | 1220 | 16×30 | 1470 | 18×35 | 1720 |
| 6800 | 13×25 | 1150 | 16×25 | 1330 | 16×35 | 1540 | 18×35 | 1830 | 22×40 | 2150 |
| 10000 | 16×25 | 1430 | 16×35 | 1680 | 18×35 | 1930 | 22×40 | 2150 | 22×50 | 2650 |
| 15000 | 16×35 | 1830 | 18×35 | 2110 | 22×40 | 2430 | 22×50 | 2750 | 25×50 | 3100 |
| 22000 | 18×40 | 2290 | 22×40 | 2650 | 22×50 | 3000 | 25×50 | 3250 |  |  |
| 33000 | 22×50 | 2800 | 22×50 | 3250 | 25×50 | 3450 |  |  |  |  |

Allowable Ripple(mA rms)at 105℃,120Hz

DIMESIONS                                                              φD×L(mm)

| Voltage Cap.(μF) | 50V |  | 63V |  | 100V |  | 160V |  | 200V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 5×11 | 1.1 |  |  | 5×11 | 1.5 |  |  |  |  |
| 0.22 | 5×11 | 2.3 |  |  | 5×11 | 3.4 |  |  |  |  |
| 0.33 | 5×11 | 3.5 |  |  | 5×11 | 5.0 |  |  |  |  |
| 0.47 | 5×11 | 7 |  |  | 5×11 | 7.0 | 6.3×12 | 8.6 | 6.3×12 | 8.6 |
| 1 | 5×11 | 12 |  |  | 5×11 | 15 | 6.3×12 | 12 | 6.3×12 | 12 |
| 2.2 | 5×11 | 18 |  |  | 5×11 | 21 | 6.3×12 | 18 | 6.3×12 | 18 |
| 3.3 | 5×11 | 25 |  |  | 5×11 | 29 | 6.3×12 | 21 | 6.3×12 | 21 |
| 4.7 | 5×11 | 29 |  |  | 5×11 | 32 | 6.3×12 | 24 | 8×12 | 28 |
| 10 | 5×11 | 46 | 5×11 | 46 | 6.3×12 | 54 | 8×12 | 41 | 10×13 | 44 |
| 22 | 5×11 | 68 | 6.3×11 | 71 | 8×12 | 93 | 10×16 | 68 | 10×20 | 71 |
| 33 | 6.3×12 | 86 | 6.3×11 | 100 | 8×12 | 130 | 10×20 | 86 | 13×20 | 100 |
| 47 | 6.3×12 | 115 | 8×12 | 120 | 10×12.5 | 165 | 13×20 | 115 | 13×20 | 115 |
| 100 | 8×12 | 190 | 10×12.5 | 215 | 10×20 | 265 | 13×25 | 170 | 16×30 | 180 |
| 220 | 10×16 | 295 | 10×20 | 335 | 13×25 | 440 | 16×35 | 270 | 18×35 | 280 |
| 330 | 10×20 | 375 | 13×20 | 510 | 16×25 | 540 | 18×40 | 350 | 22×40 | 570 |
| 470 | 13×20 | 530 | 13×25 | 640 | 16×30 | 715 | 22×40 | 610 | 22×50 | 790 |
| 1000 | 16×25 | 790 | 16×30 | 930 | 18×40 | 985 | 25×50 | 930 |  |  |
| 2200 | 16×35 | 1220 | 18×40 | 1650 | 22×50 | 1750 |  |  |  |  |
| 3300 | 18×40 | 1580 | 22×40 | 1950 | 25×50 | 2070 |  |  |  |  |
| 4700 | 22×40 | 2100 | 22×50 | 2450 |  |  |  |  |  |  |
| 6800 | 22×50 | 2500 | 25×50 | 2800 |  |  |  |  |  |  |
| 10000 | 25×50 | 2850 |  |  |  |  |  |  |  |  |

DIMESIONS                                                     φD×L(mm)

| Voltage Cap.(μF) | 250V |  | 350V |  | 400V |  | 450V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.47 | 6.3×12 | 8.6 |  |  |  |  |  |  |
| 1 | 6.3×12 | 12 | 6.3×12 | 13 | 8×12 | 13 | 8×12 | 13 |
| 2.2 | 6.3×12 | 18 | 8×12 | 18 | 8×12.5 | 20 | 8×12.5 | 20 |
| 3.3 | 8×12 | 24 | 10×12.5 | 23 | 8×12.5 | 23 | 10×13 | 25 |
| 4.7 | 8×12 | 28 | 10×12.5 | 28 | 8×12.5 | 30 | 10×13 | 32 |
| 10 | 10×16 | 46 | 10×20 | 47 | 10×17 | 50 | 13×20 | 50 |
| 22 | 13×20 | 79 | 13×25 | 79 | 13×21 | 86 | 16×25 | 86 |
| 33 | 13×20 | 100 | 16×25 | 100 | 16×30 | 110 | 16×35 | 120 |
| 47 | 13×25 | 120 | 16×35 | 120 | 16×35 | 120 | 18×40 | 130 |
| 100 | 16×30 | 180 | 18×40 | 180 | 22×40 | 250 | 22×40 | 250 |
| 220 | 22×40 | 310 | 22×50 | 500 | 25×50 | 540 |  |  |
| 330 | 22×50 | 640 |  |  |  |  |  |  |
| 470 | 25×50 | 860 |  |  |  |  |  |  |

Allowable Ripple(mA rms)at 105℃,120 Hz

在105℃,120 Hz下允许的最大纹波电流(mA rms)