# FEATURES

1. Directly mountable on printed circuit board without holders. 安装方便可靠.

2. Smaller low profile sizes than ordinary capacitors. 比普通电容的体积更小.

3. Terminal spacing fixed at 10mm for PC board plug in. 焊针距离为10mm.

4. Aluminum case designed explosion-proof vent.  铝壳底部有防爆阀.

# SPECIFICATIONS

| Item 项目 | Performance characteristics 主要特性 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated Working Voltage Range 额定工作电压范围 | 16 to 250V |  |  |  |  |  |  | 400 to 450V DC |  |  |  |  |  |  |
| Operation Temperature Range 使用温度范围 | -40 to +85℃ |  |  |  |  |  |  | -25 to +85℃ |  |  |  |  |  |  |
| Nominal Capacitance Range 标称容量范围 | 56 to 56000μF |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capacitance Tolerance 电容量允许偏差 | ±20%(120Hz,+20℃) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leakage Current 漏电流 | 1≤3√CV (μA) after 2 minutes application of rated working voltage at +20℃ 20℃.升到额定电压后两分钟后读数. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dissipation Factor tanδ (120Hz,+20℃) 损耗角正切值tanδ | WV | 10 | 16 |  | 25 | 35 | 50 |  | 63 | 80 | 100 |  | 160~250 | 400~450 |
|  | tanδ(max.) | 0.55 | 0.50 |  | 0.40 | 0.35 | 0.35 |  | 0.30 | 0.25 | 0.20 |  | 0.15 | 0.20 |
|  | For capacitance value >33000μF, add following calculated value:(rated capacitance)- 33000μF *0.1 1000μF |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Low Temperature Characteristics 低温特性 | Impedance ratio max. at 120HZ. 120HZ.时的最大阻抗比率. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Working voltage (v) |  |  | 16~100 |  |  |  | 160~250 |  |  |  | 400~450 |  |  |
|  | Z-25℃/Z+20℃ |  |  | 4 |  |  |  | 3 |  |  |  | 8 |  |  |
|  | Z-40℃/Z+20℃ |  |  | 15 |  |  |  | 12 |  |  |  | - |  |  |
| High Temperature Loading 高温负荷 | Test conditions 测试条件. The following specifications shall be satisified when the capacitors are restored to 20℃after the rated voltage applied for 2000 hours at 85℃. 85℃,施加额定电压2000小时后,经过不少于16小时室温恢复,应满足以下要求: Leakage current : ≤Initial specified value Capacitance change : ≤±20% of initial measured value tan δ : ≤200% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shelf Life 高温贮存 | At 85℃ no voltage applied after 1000 hours the capacitors shall meet the following limite. 85 ℃ 1000小时(不加电压)后, ,经过不少于16小时室温恢复,应满足以下要求: Leakage current : ≤200% of Initial specified value Capacitance change : ≤±15% of initial measured value tan δ : ≤150% of initial specified value |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Others 其它 | JIS C-5141 JIS C-5102 |  |  |  |  |  |  |  |  |  |  |  |  |  |

1) Maximum rms ripple current at 120HZ, 85℃ are give in the table.

120HZ, 85℃时最大纹波电流见下表.

2) Temperature multiplying factor: 温度调整因子

Where capacitors are operated at temperature other than 85℃, the

maximum ripple current must be multiplied by the figure shown in the

table below.

当电容器工作温度不为85℃时，其最大纹波电流与温度的对应关系因子如下：

Temperature coefficient  温度因子

| Temperature(℃) | 20~45 | 65 | 75 | 85 |
| --- | --- | --- | --- | --- |
| Factor | 1.37 | 1.25 | 1.18 | 1.0 |

3) Frequency multiplying factor: 频率调整系数.

If capacitors are used to filter circuits at a frequency other than 120HZ,the

Maximum ripple current msut be multiplied by the figure shown in the table below.

当电容器应用于电路频率不为120HZ时，其最大微波电流与频率的对应调整系数如下：

# Frequency coefficient

| Frequency (HZ) | 60 | 120 | 1k | 10~50K |
| --- | --- | --- | --- | --- |
| 25~100V | 0.9 | 1.0 | 1.15 | 1.25 |
| 160~250V | 0.8 | 1.0 | 1.15 | 1.47 |
| 350~450V | 0.8 | 1.0 | 1.15 | 1.47 |

DIMESIONS                                       ΦD×L (mm)

| Voltage Cap.(μF) | 16V |  |  |  | 25V |  |  |  | 35V |  |  |  | 50V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3300 |  |  |  |  |  |  |  |  |  |  |  |  | 22×30 | 25×25 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 2.35 | 2.35 |  |  |
| 3900 |  |  |  |  |  |  |  |  | 22×25 |  |  |  | 22×35 | 25×30 |  |  |
|  |  |  |  |  |  |  |  |  | 2.16 |  |  |  | 2.66 | 2.68 |  |  |
| 4700 |  |  |  |  |  |  |  |  | 22×30 | 22×25 |  |  | 22×40 | 25×35 | 25×30 | 35×25 |
|  |  |  |  |  |  |  |  |  | 2.42 | 2.42 |  |  | 3.02 | 3.07 | 2.98 | 3.30 |
| 5600 |  |  |  |  | 22×25 |  |  |  | 22×35 |  |  |  | 22×45 | 25×40 | 30×30 |  |
|  |  |  |  |  | 2.11 |  |  |  | 2.66 |  |  |  | 3.40 | 3.47 | 3.42 |  |
| 6800 |  |  |  |  | 22×30 | 25×25 |  |  | 22×40 | 25×30 | 25×30 |  | 22×50 | 25×40 | 30×35 |  |
|  |  |  |  |  | 2.47 | 2.47 |  |  | 2.97 | 2.82 | 2.93 |  | 3.84 | 3.74 | 3.93 |  |
| 8200 | 22×25 |  |  |  | 22×35 |  |  |  | 22×45 | 25×35 |  |  |  | 25×50 | 30×40 | 35×30 |
|  | 2.55 |  |  |  | 2.86 |  |  |  | 3.29 | 3.17 |  |  |  | 4.44 | 4.47 | 4.36 |
| 10000 | 22×30 |  |  |  | 22×40 | 25×30 | 30×25 |  | 22×50 | 25×40 | 30×30 |  |  |  |  | 35×35 |
|  | 2.89 |  |  |  | 3.31 | 3.15 | 3.27 |  | 3.75 | 3.65 | 3.60 |  |  |  |  | 5.01 |
| 12000 | 22×30 | 25×25 |  |  | 22×45 | 25×35 | 30×30 |  |  | 25×45 | 30×35 | 35×30 |  |  |  | 35×40 |
|  | 3.01 | 3.01 |  |  | 3.77 | 3.63 | 3.80 |  |  | 4.15 | 4.14 | 4.27 |  |  |  | 5.69 |
| 15000 | 22×35 | 22×30 | 30×25 |  | 22×50 | 25×40 |  |  |  |  | 30×40 | 35×35 |  |  |  | 35×45 |
|  | 3.45 | 3.48 | 3.61 |  | 4.21 | 4.10 |  |  |  |  | 4.77 | 4.95 |  |  |  | 6.56 |
| 18000 | 22×40 | 25×35 |  |  |  | 25×45 | 30×35 | 35×30 |  |  | 30×45 | 35×40 |  |  |  | 35×50 |
|  | 3.84 | 3.91 |  |  |  | 4.53 | 4.52 | 4.66 |  |  | 5.30 | 5.52 |  |  |  | 7.14 |
| 22000 | 22×50 | 25×40 | 30×30 |  |  |  | 30×45 | 35×35 |  |  |  | 35×45 |  |  |  |  |
|  | 4.52 | 4.40 | 4.34 |  |  |  | 5.33 | 5.26 |  |  |  | 6.20 |  |  |  |  |
| 27000 |  | 25×45 | 30×35 |  |  |  | 30×50 | 35×40 |  |  |  | 35×50 |  |  |  |  |
|  |  | 4.96 | 4.95 |  |  |  | 5.96 | 5.93 |  |  |  | 6.89 |  |  |  |  |
| 33000 |  |  | 30×40 | 35×30 |  |  |  | 35×45 |  |  |  |  |  |  |  |  |
|  |  |  | 5.60 | 5.40 |  |  |  | 6.65 |  |  |  |  |  |  |  |  |
| 39000 |  |  | 30×45 | 35×35 |  |  |  | 35×50 |  |  |  |  |  |  |  |  |
|  |  |  | 6.21 | 6.12 |  |  |  | 7.31 |  |  |  |  |  |  |  |  |
| 47000 |  |  | 30×50 | 35×40 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 6.93 | 6.89 |  |  |  |  |  |  |  |  |  |  |  |  |
| 56000 |  |  |  | 35×45 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 7.69 |  |  |  |  |  |  |  |  |  |  |  |  |

DIMESIONS                                                                      ΦD×L (mm)

| Voltage Cap.(μF) | 63V |  |  |  | 80V |  |  |  | 100V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1200 |  |  |  |  | 22×25 |  |  |  | 22×35 | 25×30 |  |  |
|  |  |  |  |  | 1.66 |  |  |  | 2.11 | 2.11 |  |  |
| 1500 |  |  |  |  | 22×30 |  |  |  | 22×40 | 25×35 | 30×25 |  |
|  |  |  |  |  | 1.96 |  |  |  | 2.45 | 2.47 | 2.56 |  |
| 1800 | 22×25 |  |  |  | 22×35 | 25×30 |  |  | 22×45 | 25×35 |  |  |
|  | 1.82 |  |  |  | 2.11 | 2.11 |  |  | 2.77 | 2.81 |  |  |
| 2200 | 22×30 | 25×25 |  |  | 22×40 | 25×30 | 30×25 |  | 22×50 | 25×40 | 30×35 |  |
|  | 2.14 | 2.14 |  |  | 2.44 | 2.46 | 2.56 |  | 3.15 | 3.21 | 3.17 |  |
| 2700 | 22×35 | 25×30 |  |  | 22×45 | 25×35 |  |  |  | 25×50 | 30×40 | 35×30 |
|  | 2.49 | 2.52 |  |  | 2.82 | 2.86 |  |  |  | 3.66 | 3.65 | 3.77 |
| 3300 | 22×40 | 25×35 | 30×25 |  | 22×50 | 25×40 | 30×35 |  |  |  | 30×45 |  |
|  | 2.72 | 2.74 | 2.84 |  | 3.23 | 3.29 | 3.25 |  |  |  | 4.18 |  |
| 3900 | 22×40 | 25×35 |  |  |  | 25×45 | 30×35 |  |  |  | 30×50 | 35×35 |
|  | 3.09 | 3.13 |  |  |  | 3.71 | 3.70 |  |  |  | 4.67 | 4.61 |
| 4700 | 22×50 | 25×40 | 30×35 | 35×25 |  | 25×50 | 30×40 | 35×35 |  |  | 30×50 | 35×40 |
|  | 3.69 | 3.59 | 3.54 | 3.25 |  | 4.20 | 4.23 | 4.12 |  |  | 5.26 | 5.23 |
| 5600 |  | 25×45 | 30×35 |  |  |  | 30×45 | 35×35 |  |  |  | 35×50 |
|  |  | 4.01 | 4.00 |  |  |  | 4.70 | 4.64 |  |  |  | 5.88 |
|  |  |  | 30×40 | 35×30 |  |  | 30×35 | 35×40 |  |  |  |  |
|  |  |  | 4.55 | 4.44 |  |  | 4.52 | 5.24 |  |  |  |  |
| 8200 |  |  | 30×45 | 35×35 |  |  |  | 35×45 |  |  |  |  |
|  |  |  | 5.12 | 5.05 |  |  |  | 5.89 |  |  |  |  |
| 10000 |  |  |  | 35×40 |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.75 |  |  |  |  |  |  |  |  |
| 12000 |  |  |  | 35×45 |  |  |  |  |  |  |  |  |
|  |  |  |  | 6.47 |  |  |  |  |  |  |  |  |

DIMESIONS                                                                      ΦD×L (mm)

| Voltage Cap.(μF) | 160V(2C) |  |  |  | 180V(2Z) |  |  |  | 200V(2D) |  |  |  | 250V(2E) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 220 |  |  |  |  |  |  |  |  |  |  |  |  | 22×30 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 1.15 |  |  |  |
| 270 |  |  |  |  |  |  |  |  | 22×30 |  |  |  | 22×35 | 25×30 |  |  |
|  |  |  |  |  |  |  |  |  | 1.25 |  |  |  | 1.25 | 1.25 |  |  |
| 330 |  |  |  |  | 22×30 |  |  |  | 22×30 |  |  |  | 22×40 | 25×35 |  |  |
|  |  |  |  |  | 1.35 |  |  |  | 1.40 |  |  |  | 1.45 | 1.45 |  |  |
| 390 | 22×30 |  |  |  | 22×30 |  |  |  | 22×35 | 25×30 |  |  | 22×45 | 25×35 | 30×30 |  |
|  | 1.55 |  |  |  | 1.55 |  |  |  | 1.60 | 1.60 |  |  | 1.70 | 1.70 | 1.70 |  |
| 470 | 22×30 | 25×25 |  |  | 22×35 | 25×30 |  |  | 22×40 | 25×30 |  |  | 22×50 | 25×40 | 30×35 |  |
|  | 1.75 | 1.75 |  |  | 1.75 | 1.75 |  |  | 1.80 | 1.80 |  |  | 1.90 | 1.90 | 1.90 |  |
| 560 | 22×35 | 25×30 |  |  | 22×40 | 25×35 |  |  | 22×45 | 25×35 | 30×30 |  |  | 25×45 | 30×35 | 35×30 |
|  | 1.90 | 1.90 |  |  | 2.00 | 2.00 |  |  | 2.00 | 2.00 | 2.00 |  |  | 2.15 | 2.15 | 2.15 |
| 680 | 22×40 | 25×35 | 30×25 |  | 22×45 | 25×35 | 30×30 |  | 22×50 | 25×40 | 30×30 |  |  | 25×50 | 30×40 | 35×30 |
|  | 2.15 | 2.15 | 2.15 |  | 2.25 | 2.25 | 2.25 |  | 2.30 | 2.25 | 2.30 |  |  | 2.34 | 2.35 | 2.35 |
| 820 | 22×45 | 25×40 | 30×30 |  | 22×50 | 25×45 | 30×35 | 35×25 |  | 25×45 | 30×35 | 35×30 |  |  | 30×45 | 35×35 |
|  | 2.45 | 2.45 | 2.45 |  | 2.50 | 2.50 | 2.50 | 2.50 |  | 2.50 | 2.50 | 2.50 |  |  | 2.75 | 2.75 |
| 1000 |  | 25×45 | 30×35 | 35×25 |  | 25×50 | 30×40 | 35×30 |  |  | 30×40 | 35×30 |  |  | 30×50 | 35×40 |
|  |  | 2.80 | 2.80 | 2.80 |  | 2.85 | 2.85 | 2.85 |  |  | 2.95 | 2.95 |  |  | 3.00 | 3.00 |
| 1200 |  | 25×50 | 30×40 | 35×30 |  |  | 30×45 | 35×35 |  |  | 30×45 | 35×35 |  |  |  | 35×45 |
|  |  | 3.25 | 3.25 | 3.25 |  |  | 3.30 | 3.30 |  |  | 3.40 | 3.40 |  |  |  | 3.50 |
| 1500 |  |  | 30×45 | 35×35 |  |  | 30×50 | 35×40 |  |  |  | 35×40 |  |  |  |  |
|  |  |  | 3.75 | 3.75 |  |  | 3.75 | 3.75 |  |  |  | 3.80 |  |  |  |  |
| 1800 |  |  | 30×50 | 35×40 |  |  |  | 35×45 |  |  |  | 35×45 |  |  |  |  |
|  |  |  | 4.00 | 4.00 |  |  |  | 4.05 |  |  |  | 4.15 |  |  |  |  |
| 2200 |  |  |  | 35×45 |  |  |  | 35×50 |  |  |  |  |  |  |  |  |
|  |  |  |  | 4.50 |  |  |  | 4.60 |  |  |  |  |  |  |  |  |
| 2700 |  |  |  | 35×50 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 5.15 |  |  |  |  |  |  |  |  |  |  |  |  |

| Voltage Cap.(μF) | 400V(2G) |  |  |  | 450V(2W) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56 |  |  |  |  | 22×25 |  |  |  |
|  |  |  |  |  | 0.59 |  |  |  |
| 68 |  |  |  |  | 22×30 |  |  |  |
|  |  |  |  |  | 0.63 |  |  |  |
| 82 | 22×25 |  |  |  | 22×30 | 25×25 |  |  |
|  | 0.72 |  |  |  | 0.78 | 0.78 |  |  |
| 100 | 22×30 |  |  |  | 22×35 | 25×30 |  |  |
|  | 0.77 |  |  |  | 0.92 | 0.92 |  |  |
| 120 | 22×35 | 25×30 |  |  | 22×40 | 25×35 | 30×25 |  |
|  | 0.94 | 0.94 |  |  | 1.06 | 1.08 | 1.05 |  |
| 150 | 22×40 | 25×30 |  |  | 22×50 | 25×40 | 30×30 |  |
|  | 1.13 | 0.92 |  |  | 1.28 | 1.25 | 1.23 |  |
| 180 | 22×45 | 25×35 | 30×30 |  |  | 25×45 | 30×40 |  |
|  | 1.27 | 1.06 | 1.26 |  |  | 1.41 | 1.48 |  |
| 220 | 22×50 | 25×40 | 30×35 | 35×25 |  | 25×50 | 30×40 | 35×30 |
|  | 1.45 | 1.22 | 1.46 | 1.50 |  | 1.59 | 1.60 | 1.57 |
| 270 |  | 25×45 | 30×35 | 35×30 |  |  | 30×45 | 35×35 |
|  |  | 1.69 | 1.68 | 1.46 |  |  | 1.83 | 1.80 |
| 330 |  | 25×50 | 30×40 | 35×30 |  |  | 30×50 | 35×40 |
|  |  | 1.92 | 1.93 | 1.88 |  |  | 2.08 | 2.06 |
| 390 |  |  | 30×45 | 35×35 |  |  |  | 35×45 |
|  |  |  | 2.16 | 2.13 |  |  |  | 2.31 |
| 470 |  |  | 30×50 | 35×40 |  |  |  | 35×50 |
|  |  |  | 2.43 | 2.42 |  |  |  | 2.47 |
| 560 |  |  |  | 35×45 |  |  |  |  |
|  |  |  |  | 2.72 |  |  |  |  |
| 680 |  |  |  | 35×50 |  |  |  |  |
|  |  |  |  | 3.08 |  |  |  |  |

DIMESIONS                                                                      ΦD×L (mm)

| Voltage Cap.(μF) | 35V |  | 50V |  |
| --- | --- | --- | --- | --- |
| 0.1 |  |  | 4×5(3×5) | 1.0 |
| 0.22 |  |  | 4×5(3×5) | 2.0 |
| 0.33 |  |  | 4×5(3×5) | 2.8 |
| 0.47 |  |  | 4×5(3×5) | 4.0 |
| 1 |  |  | 4×5(3×5) | 8.4 |
| 2.2 | 3×5 | 8.4 | 4×5 | 13 |
| 3.3 | 4×5 | 15 | 4×5 | 17 |
| 4.7 | 4×5 | 18 | 5×5 | 20 |
| 10 | 5×5 | 29 | 6.3×5 | 33 |
| 22 | 6.3×5 | 46 | 8×5 | 52 |
| 33 | 8×5 | 62 | 8×5 | 71 |
| 47 | 8×5 | 80 |  |  |
| 100 |  |  |  |  |
| 220 |  |  |  |  |
| 330 |  |  |  |  |

Maximum Allowable Ripple Current (mA rms)at 85℃,120Hz